const int trigPin = 9;

const int echoPin = 10;

const int redLED = 13;

const int yellowLED = 12;

const int greenLED = 11;

const int buttonPin = 2;

const unsigned long debounceTime = 50;

const unsigned long echoTimeout = 30000;

bool buttonState = false;

bool lastButtonReading = HIGH;

unsigned long lastDebounceTime = 0;

unsigned long lastMeasurementTime = 0;

float distanceSamples[3];

int sampleCount = 0;

void turnOffLEDs() {

digitalWrite(redLED, LOW);

digitalWrite(yellowLED, LOW);

digitalWrite(greenLED, LOW);

}

void updateButton() {

bool reading = digitalRead(buttonPin);

if (reading != lastButtonReading) {

lastDebounceTime = millis();

}

if (millis() - lastDebounceTime >= debounceTime) {

buttonState = (reading == LOW);

}

lastButtonReading = reading;

}

float getMedian(float a, float b, float c) {

if (a > b) {

float temp = a;

a = b;

b = temp;

}

if (b > c) {

float temp = b;

b = c;

c = temp;

}

if (a > b) {

float temp = a;

a = b;

b = temp;

}

return b;

}

bool measureDistance(float &distance) {

digitalWrite(trigPin, LOW);

delayMicroseconds(2);

digitalWrite(trigPin, HIGH);

delayMicroseconds(10);

digitalWrite(trigPin, LOW);

unsigned long duration = pulseIn(echoPin, HIGH, echoTimeout);

if (duration == 0) {

return false;

}

distance = duration * 0.0343 / 2;

return true;

}

void setup() {

pinMode(trigPin, OUTPUT);

pinMode(echoPin, INPUT);

pinMode(redLED, OUTPUT);

pinMode(yellowLED, OUTPUT);

pinMode(greenLED, OUTPUT);

pinMode(buttonPin, INPUT_PULLUP);

Serial.begin(9600);

turnOffLEDs();

Serial.println("System ready");

}

void loop() {

updateButton();

if (!buttonState) {

turnOffLEDs();

sampleCount = 0;

return;

}

if (millis() - lastMeasurementTime < 100) {

return;

}

lastMeasurementTime = millis();

float distance;

bool validMeasurement = measureDistance(distance);

if (!validMeasurement) {

Serial.println("ERROR: No Echo");

turnOffLEDs();

sampleCount = 0;

return;

}

distanceSamples[sampleCount] = distance;

sampleCount++;

if (sampleCount < 3) {

return;

}

float filteredDistance = getMedian(

distanceSamples[0],

distanceSamples[1],

distanceSamples[2]

);

sampleCount = 0;

turnOffLEDs();

if (filteredDistance < 20) {

digitalWrite(redLED, HIGH);

}

else if (filteredDistance <= 50) {

digitalWrite(yellowLED, HIGH);

}

else {

digitalWrite(greenLED, HIGH);

}

Serial.print("Distance: ");

Serial.print(filteredDistance);

Serial.println(" cm");

}